# MOIRA’S DESKTOP REFERENCE # USING 2182 kHz WITH VOICE - Push [2182kHz] to select the emergency frequency. - Push [ALARM] and [TX FREQ] for 1 sec to transmit a 2-tone alarm signal for at least 30 sec. The transceiver automatically stops the alarm after 50 sec. - Push [ALARM] to turn the alarm transmisison off, then push and hold the PTT switch on the microphone and send the following information: - “MAYDAY, MAYDAY, MAYDAY.” - “THIS IS ….” (name of vessel) - “LOCATED AT ….” (vessel’s posiiton) - Give the reason for the distress call. - Explain what assistance you need. - Give additional information: - Vessel type - Vessel length - Vessel color - Number of people onboard

| 90% charge on 360Ah bank |  |
| --- | --- |
| 13.6V | < 14A |
| 13.5 | 12 |
| 13.4 | 10.4 |
| 13.2 | 7.2 |
| 100 % charge on 360Ah bank |  |
| 14.0V | < 5.4A |
| 13.8 | 4.5 |
| 13.6 | 3.6 |
| 13.2 | 1.8 |
| 12.8 | 0 |
| Resting voltage (Gel) |  |
| 100% | 12.8V |
| 75 | 12.5 |
| 50 | 12.3 |
| 25 | 12.0 |
| 0 | 11.8 |

# PROGRAMMING A CHANNEL - Select channel - Press CE to show frequency - Enter frequency with keypad - If necessary select correct MODE - Press and hold RX 1 sec. - Press TX so “TX” flashes - Press CE to show frequency - Enter frequency with keypad - Press and hold TX 1 sec. - Press TX to stop “TX” flashing

| # SAFETY CHANNELS |  |
| --- | --- |
| # Channel # | # Frequency |
| # SAFETY |  |
| # 1 | # 2182 kHz |
| # 2 | # 4125 |
| # 3 | # 6215 |
| # 4 | # 8291 |
| # 5 | # 12290 |
| # 6 | # 16420 |
| # USCG |  |
| # 7 | # 2670 |
| # 8 | # (RX/TX) # 4426/4134 |
| # 9 | # 6501/6200 |
| # 10 | # 8764/8240 |
| # 11 | # 13089/12242 |
| # 12 | # 17314/16432 |
| # MMSN |  |
| # 147 | # 14300 |
| # USCG RCC’s |  |
| # Miami | # (305)415-6800 |
| # San Juan | # (787)289-2042 |
| # Portsmouth | # (757)398-6231 |

| ## Geostrophic Wind Force (after Dashew P.113) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ## Wind speed (Kts) given latitude and spacing in degrees between 4mb isobars |  |  |  |  |  |
| Degrees Between 4mb Isobars | Latitude of observation |  |  |  |  |
|  | 10 | 20 | 30 | 40 | 50 |
| 1 | -- | 78 G 116 | 54 G 80 | 42 G 62 | 36 G 52 |
| 2 | 76 G 115 | 39 G 58 | 27 G 40 | 21 G 31 | 18 G 26 |
| 3 | 50 G 75 | 26 G 39 | 18 G 26 | 14 G 20 | 12 G 18 |
| 4 | 36 G 55 | 19 G 30 | 14 G 20 | 10 G 15 | 9 G 13 |
| 5 | 29 G 44 | 16 G 23 | 10 G 16 | 8 G 13 | 7 G 10 |

| Charge curve on 360Ah | 90% Charge | 100% Charge |
| --- | --- | --- |
| 14.0V | < 18A | < 5.4A |
| 13.8 | 16 | 4.5 |
| 13.6 | 14 | 3.6 |
| 13.5 | 12 | 3.1 |
| 13.4 | 10 | 2.7 |
| 13.2 | 7.2 | 1.8 |
| 12.8 | 3.6 | 0 |
| Resting voltage (Gel) |  |  |
| 100% | 12.8V |  |
| 75 | 12.5 |  |
| 50 | 12.3 |  |
| 25 | 12.0 |  |
| 0 | 11.8 |  |

Leonard, 5.5% bleach (“Clorox”) - Wash vegs: 1 oz/gal - Clean tanks 6-8oz/10-15 gal then rinse 3x - Treat water: 1 1/3-1 2/3 oz/33 gal

D’Antonio 2 cycle oil in diesel: 1/3oz/gal

| Speed to Exceed |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Deviation (Degrees) |  |  |  |  |  |
| Speed Direct | 10 | 20 | 30 | 40 | 50 | 60 |
| 2 | 2.0 | 2.1 | 2.3 | 2.6 | 3.1 | 4.0 |
| 2.5 | 2.5 | 2.7 | 2.9 | 3.3 | 3.9 | 5.0 |
| 3 | 3.0 | 3.2 | 3.5 | 3.9 | 4.7 | 6.0 |
| 3.5 | 3.6 | 3.7 | 4.0 | 4.6 | 5.4 | 7.0 |
| 4 | 4.1 | 4.3 | 4.6 | 5.2 | 6.2 | 8.0 |
| 4.5 | 4.6 | 4.8 | 5.2 | 5.9 | 7.0 | 9.0 |
| 5 | 5.1 | 5.3 | 5.8 | 6.5 | 7.8 | 10.0 |
| 5.5 | 5.6 | 5.9 | 6.4 | 7.2 | 8.6 | 11.0 |
| 6 | 6.1 | 6.4 | 6.9 | 7.8 | 9.3 | 12.0 |
| 6.5 | 6.6 | 6.9 | 7.5 | 8.5 | 10.1 | 13.0 |
| 7 | 7.1 | 7.4 | 8.1 | 9.1 | 10.9 | 14.0 |
| 7.5 | 7.6 | 8.0 | 8.7 | 9.8 | 11.7 | 15.0 |